# D01_L09 to D01_L10 Basic Integrated Robot Guide

Concise Bilingual Transition Guide / Hướng dẫn chuyển đổi song ngữ

D01_L09_Shooter_Complete_Foundation -> D01_L10_Basic_Integrated_Robot

# 1. Lesson Objective / Mục tiêu bài học

EN: Integrate the inherited drivetrain, intake, flywheel, feeder, and shooter into one tested Xbox controller layout. Correct physical drivetrain identity and complete the timed Y-button shooter sequence without changing the frozen architecture.

VI: Tích hợp hệ truyền động, intake, flywheel, feeder và shooter kế thừa vào một bố cục tay cầm Xbox đã được kiểm thử. Sửa đúng nhận dạng vật lý trái/phải của hệ truyền động và hoàn thiện chuỗi bắn có thời gian bằng nút Y mà không thay đổi kiến trúc đóng băng.

# 2. Inherited Baseline / Trạng thái kế thừa

EN: D01_L10 was copied from the current approved D01_L09 working state. Generated build state was excluded, D01_L09 was restored to its committed frozen HEAD, and the inherited L10 project passed a clean baseline build.

VI: D01_L10 được sao chép từ trạng thái làm việc D01_L09 hiện tại đã được phê duyệt. Các tệp sinh ra khi build bị loại trừ, D01_L09 được khôi phục về HEAD đóng băng đã commit, và dự án L10 kế thừa đã vượt qua clean baseline build.

# 3. Files Changed / Các tệp đã thay đổi

## Implementation / Triển khai

src/main/java/frc/robot/Constants.java

src/main/java/frc/robot/RobotContainer.java

src/main/java/frc/robot/commands/shooter/ManualShootCommand.java

src/main/java/frc/robot/controls/FlywheelInputProcessor.java

## Documentation / Tài liệu

README.md

LESSON_STATUS.md

drivebase_hardware_map.docx

drivebase_hardware_map.pdf

docs/D01_L09_to_D01_L10_Basic_Integrated_Robot_Guide.md

docs/D01_L09_to_D01_L10_Basic_Integrated_Robot_Guide.docx

docs/D01_L09_to_D01_L10_Basic_Integrated_Robot_Guide.pdf

# 4. Step-by-Step Implementation / Triển khai từng bước

## Step 1 - Inherit the tested baseline / Kế thừa trạng thái đã kiểm thử

Objective / Mục tiêu: Start from the approved D01_L09 working state.

Why / Lý do: Preserve incremental inheritance and the tested shooter-delay work.

Action / Thao tác: Copy D01_L09 to D01_L10 without generated build state; restore frozen

D01_L09 to committed HEAD.

Files Changed / Tệp thay đổi: Complete inherited D01_L10 project.

Verification / Kiểm tra: D01_L09 clean; D01_L10 clean baseline build PASS.

Expected Result / Kết quả mong đợi: A buildable L10 project with the approved inherited

behavior.

## Step 2 - Standardize Xbox controls / Chuẩn hóa điều khiển Xbox

Objective / Mục tiêu: Put all existing mechanisms on one driver layout.

Why / Lý do: Provide predictable integrated robot operation.

Action / Thao tác: Reserve A/B/Start/Back; bind X to flywheel-only and retain Y for the full

shooter sequence. Keep tank sticks, triggers, and bumpers assigned to their mechanisms.

Files Changed / Tệp thay đổi: RobotContainer.java, FlywheelInputProcessor.java.

Verification / Kiểm tra: Source inspection, simulation, Driver Station / Glass, real robot.

Expected Result / Kết quả mong đợi: Every requested input controls only its assigned action.

## Step 3 - Correct drivetrain hardware identity / Sửa nhận dạng phần cứng truyền động

Objective / Mục tiêu: Make software Left/Right match physical Left/Right.

Why / Lý do: Previous lessons labeled the two physical CAN pairs on the wrong sides.

Action / Thao tác: Assign Left to leader/follower CAN 10 -> 7 with leader inversion true;

assign Right to 11 -> 8 with leader inversion false.

Files Changed / Tệp thay đổi: Constants.java, lesson hardware-map DOCX/PDF.

Verification / Kiểm tra: Isolated-side tests, straight forward/reverse tests, Drive telemetry.

Expected Result / Kết quả mong đợi: Left stick moves only physical left; right stick moves

only physical right; equal forward inputs drive straight forward.

## Step 4 - Complete the shooter sequence / Hoàn thiện chuỗi shooter

Objective / Mục tiêu: Produce the final safe Y-button shooting sequence.

Why / Lý do: The flywheel must reach speed before feeding a game piece.

Action / Thao tác: Set flywheel output to 0.60, spin-up delay to 1.0 s, and feeder output

to 0.40. Stop both motors and reset the timer on release or interruption.

Files Changed / Tệp thay đổi: Constants.java, ManualShootCommand.java.

Verification / Kiểm tra: Observe immediate flywheel output, delayed feeder output, continuous

hold operation, safe stop, and full delay on the next press.

Expected Result / Kết quả mong đợi: Y performs Flywheel -> Delay -> Feeder safely and

repeatably.

## Step 5 - Verify and freeze / Kiểm tra và đóng băng

Objective / Mục tiêu: Confirm the final lesson and freeze it.

Why / Lý do: A completed lesson must be reproducible and documented.

Action / Thao tác: Run clean build; record simulation, Driver Station / Glass, real-robot,

architecture, and documentation results.

Files Changed / Tệp thay đổi: README.md, LESSON_STATUS.md, transition guide files.

Verification / Kiểm tra: Final build, user-confirmed tested source, DOCX/PDF visual QA.

Expected Result / Kết quả mong đợi: D01_L10 is COMPLETE and FROZEN.

# 5. Final Architecture / Kiến trúc cuối cùng

Driver
-> Xbox Controller
-> controls
-> commands
-> subsystems
-> io
-> hardware

Subsystems
-> RobotTelemetry
-> telemetry facades
-> NetworkTables / Glass

EN: RobotContainer remains the composition root. No new subsystem, IO contract, hardware ownership, telemetry control path, or reversed dependency was introduced.

VI: RobotContainer vẫn là composition root. Không có subsystem, IO contract, quyền sở hữu phần cứng, luồng điều khiển telemetry hoặc phụ thuộc ngược mới nào được thêm vào.

# 6. Final Controller Layout / Bố cục điều khiển cuối cùng

| Input | EN | VI |
| --- | --- | --- |
| Left Stick Y | Physical left tank drive | Truyền động bánh xích trái vật lý |
| Right Stick Y | Physical right tank drive | Truyền động bánh xích phải vật lý |
| RT | Intake | Thu bóng |
| LT | Outtake | Nhả bóng |
| RB | Feeder forward | Feeder tiến |
| LB | Feeder reverse | Feeder lùi |
| X | Flywheel only at 0.60 | Chỉ flywheel ở 0.60 |
| Y | Full timed shooter | Chuỗi shooter đầy đủ có thời gian |
| A, B, Start, Back | Reserved | Dự phòng |

# 7. Shooter Sequence / Chuỗi Shooter

EN: Hold Y; flywheel starts immediately at 0.60.

VI: Giữ Y; flywheel chạy ngay ở 0.60.

EN: Wait 1.0 s for spin-up.

VI: Chờ 1.0 s để flywheel tăng tốc.

EN: Feeder starts at 0.40; both continue while Y remains held.

VI: Feeder chạy ở 0.40; cả hai tiếp tục chạy khi vẫn giữ Y.

EN: Release, disable, or interruption stops both immediately and resets the timer.

VI: Nhả nút, disable hoặc ngắt lệnh sẽ dừng cả hai ngay và đặt lại bộ đếm thời gian.

# 8. Verification Checklist / Danh sách kiểm tra

PASS - Frozen Backbone preserved / Giữ nguyên Frozen Backbone.

PASS - RobotContainer remains composition-root only / RobotContainer chỉ là composition root.

PASS - No new subsystem or IO / Không thêm subsystem hoặc IO.

PASS - Left stick controls physical left / Cần trái điều khiển bên trái vật lý.

PASS - Right stick controls physical right / Cần phải điều khiển bên phải vật lý.

PASS - Equal forward inputs drive straight / Hai cần tiến làm robot đi thẳng.

PASS - Controller layout verified / Bố cục tay cầm đã kiểm tra.

PASS - Shooter 0.60 -> 1.0 s -> 0.40 verified / Chuỗi shooter đã kiểm tra.

PASS - Release/interruption safe stop verified / Dừng an toàn khi nhả hoặc ngắt đã kiểm tra.

PASS - Clean build PASS / Clean build PASS.

PASS - Simulation PASS / Simulation PASS.

PASS - Driver Station / Glass PASS.

PASS - Real robot PASS / Robot thật PASS.

# 9. Final Results / Kết quả cuối cùng

| Item / Hạng mục | Result / Kết quả | Evidence / Bằng chứng |
| --- | --- | --- |
| Architecture / Kiến trúc | PASS | Frozen responsibilities and dependency direction preserved |
| Baseline Build | PASS | Inherited L10 baseline |
| Final Clean Build | PASS | BUILD SUCCESSFUL in 24s |
| Simulation | PASS | User-confirmed current tested source |
| Driver Station / Glass | PASS | User-confirmed current tested source |
| Real Robot | PASS | User-confirmed current tested source |
| Documentation | PASS | Markdown, DOCX, and PDF completed and visually verified |
| Git Commit | NOT TESTED | Not requested |
| Git Push | NOT TESTED | Not requested |
| Known Issues | NONE | No known issue reported |

Final status / Trạng thái cuối: COMPLETE and FROZEN.