# Drivebase Hardware Map

Project: FRC Java Coding Lab 7.0 - Real Robot Programming
Lesson: D01_L10_Basic_Integrated_Robot
Version: 1.1
Author: SSIS
Mentor: SSIS

## Purpose

This document records the current drivetrain hardware configuration and serves as the single source of truth for the drivebase.

## Drivebase Overview

| Item | Value |
| --- | --- |
| Drive Type | Tank Drive |
| Motor Controller | REV SPARK MAX |
| Motor Type | Brushed CIM |
| Motor Count | 4 |
| Power Distribution | REV PDH |
| Main Controller | RoboRIO 2 |
| Framework | WPILib 2026 |

## Drivebase CAN Mapping

| CAN ID | Device | Role | Motor | Notes |
| --- | --- | --- | --- | --- |
| 10 | SPARK MAX | Left Leader | Left CIM | Main left drivetrain controller |
| 7 | SPARK MAX | Left Follower | Left CIM | Follows CAN 10 |
| 11 | SPARK MAX | Right Leader | Right CIM | Main right drivetrain controller |
| 8 | SPARK MAX | Right Follower | Right CIM | Follows CAN 11 |

## Leader / Follower

Left:  CAN 10 (Leader) -> CAN 7 (Follower)
Right: CAN 11 (Leader) -> CAN 8 (Follower)

## Motor Configuration

| Setting | Value |
| --- | --- |
| Current Limit | 60 A |
| Voltage Compensation | 12.0 V |
| Left Leader Inverted | TRUE |
| Right Leader Inverted | FALSE |
| Follower Mode | Enabled |

## Controller Mapping

| Button | Function |
| --- | --- |
| Left Stick Y | Left Tank Drive |
| Right Stick Y | Right Tank Drive |
| RT | Intake |
| LT | Outtake |
| RB | Feed |
| LB | Reverse Feed |
| X | Flywheel Only |
| Y | Full Shooter: Flywheel 0.60 -> 1.0 s Spin-up Delay -> Feeder 0.40 |
| A, B, Start, Back | Reserved |

## Software Flow

Xbox Controller → RobotContainer → DriveSubsystem → DriveIO → DriveIOSparkMax → Leaders → Followers → Wheels

## Verification

CAN 10 Left Leader ID VERIFIED

CAN 7 Left Follower ID VERIFIED

CAN 11 Right Leader ID VERIFIED

CAN 8 Right Follower ID VERIFIED

Clean Build PASS

Simulation PASS

Driver Station / Glass PASS

Real Robot PASS

## D01_L10 Hardware Identity Correction

Previous lessons used an incorrect physical Left/Right drivetrain identity. D01_L10 corrects the hardware mapping so software Left controls the physical left drivetrain and software Right controls the physical right drivetrain. No command, subsystem, IO contract, or telemetry architecture was changed.