# Robot Hardware Map

FRC Java Coding Lab 7.0 - Real Robot Programming

Updated from Phoenix Tuner X verification - 2026-07-29

# 1. Drivebase

| Item | Verified / Current Value |
| --- | --- |
| Drive type | Tank Drive |
| Motor controllers | 4 x REV SPARK MAX |
| Motors | 4 x brushed CIM |
| Left leader / follower | CAN 11 -> CAN 8 |
| Right leader / follower | CAN 10 -> CAN 7 |
| Current limit | 60 A |
| Voltage compensation | 12 V |
| Encoder | Leader integrated relative encoder |

# 2. Intake

| Item | Verified / Current Value |
| --- | --- |
| Motor | Kraken X60 |
| Integrated controller | CTRE Talon FX |
| CAN ID | 1 |
| CAN bus | rio |
| Phoenix 6 application version | 26.2.0.0 |
| Device state | Running Application |
| Firmware status | Update available |
| Software API | CTRE Phoenix 6 TalonFX |

# 3. Shooter Flywheel

| Item | Verified / Current Value |
| --- | --- |
| Motor | Kraken X60 |
| Integrated controller | CTRE Talon FX |
| CAN ID | 9 |
| CAN bus | rio |
| Phoenix 6 application version | 26.2.0.0 |
| Device state | Running Application |
| Firmware status | Update available |
| Software API | CTRE Phoenix 6 TalonFX |

# 4. External Sensor

| Item | Verified / Current Value |
| --- | --- |
| Device | CTRE CANcoder |
| CAN ID | 4 |
| CAN bus | rio |
| Phoenix 6 application version | 26.1.0.0 |
| Device state | Running Application |
| Software API | CTRE Phoenix 6 CANcoder |

# 5. Loader / Additional Mechanism

| Item | Verified / Current Value |
| --- | --- |
| Status | Not confirmed in the supplied Phoenix Tuner X detection view |
| Previous map entry | REV SPARK MAX, CAN 19 |
| Authority | Do not treat CAN 19 as verified until the physical device is detected and identified |

# 6. Verified CAN Device Summary

| Function | Device | Vendor | CAN ID | CAN Bus |
| --- | --- | --- | --- | --- |
| Intake | Kraken X60 / Talon FX | CTRE | 1 | rio |
| Shooter Flywheel | Kraken X60 / Talon FX | CTRE | 9 | rio |
| External Sensor | CANcoder | CTRE | 4 | rio |
| Drive Left Leader | SPARK MAX | REV | 11 | rio |
| Drive Left Follower | SPARK MAX | REV | 8 | rio |
| Drive Right Leader | SPARK MAX | REV | 10 | rio |
| Drive Right Follower | SPARK MAX | REV | 7 | rio |

Implementation authority: Phoenix Tuner X detection is authoritative for the CTRE devices shown above. The legacy hardware map entries that identified the Intake and Shooter Flywheel as REV SPARK MAX devices are superseded.

Required code mapping: Intake must use CTRE Phoenix 6 TalonFX on CAN ID 1. Shooter Flywheel must use CTRE Phoenix 6 TalonFX on CAN ID 9. Do not instantiate REV SparkMax objects for these two mechanisms.