# Drivebase Hardware Map

Project: FRC Java Coding Lab 7.0 – Real Robot Programming
Module: D00 – Competition Robot Foundation
Version: 1.0
Author: SSIS
Mentor: SSIS

## Purpose

This document records the current drivetrain hardware configuration and serves as the single source of truth for the drivebase.

## Drivebase Overview

| Item | Value |
| --- | --- |
| Drive Type | Tank Drive |
| Motor Controller | REV SPARK MAX |
| Motor Type | Brushed CIM |
| Motor Count | 4 |
| Power Distribution | REV PDH |
| Main Controller | RoboRIO 2 |
| Framework | WPILib 2026 |

## Drivebase CAN Mapping

| CAN ID | Device | Role | Motor | Notes |
| --- | --- | --- | --- | --- |
| 11 | SPARK MAX | Left Leader | Left CIM | Main left drivetrain controller |
| 8 | SPARK MAX | Left Follower | Left CIM | Follows CAN 11 |
| 10 | SPARK MAX | Right Leader | Right CIM | Main right drivetrain controller |
| 7 | SPARK MAX | Right Follower | Right CIM | Follows CAN 10 |

## Leader / Follower

Left:  CAN 11 (Leader)  →  CAN 8 (Follower)
Right: CAN 10 (Leader) → CAN 7 (Follower)

## Motor Configuration

| Setting | Value |
| --- | --- |
| Current Limit | 60 A |
| Voltage Compensation | 12.0 V |
| Left Leader Inverted | TRUE |
| Right Leader Inverted | FALSE |
| Follower Mode | Enabled |

## Controller Mapping

| Button | Function |
| --- | --- |
| A | Drive Forward (50%) |
| B | Drive Reverse (50%) |
| Release | Stop Drivebase |

## Software Flow

Xbox Controller → RobotContainer → DriveSubsystem → DriveIO → DriveIOSparkMax → Leaders → Followers → Wheels

## Verification

CAN 11 PASS

CAN 8 PASS

CAN 10 PASS

CAN 7 PASS

Build PASS

Deploy PASS

Forward Test PASS

Reverse Test PASS