# FRC ROBOT PROJECT - FINAL FROZEN BACKBONE ARCHITECTURE

FRC Java Coding Lab 7.0 - Permanent Inheritance Development Contract

Driver -> Xbox Controller -> controls -> commands -> subsystems -> io -> Hardware

telemetry observes beside the pipeline; it never controls the pipeline

| PACKAGE TREE | RESPONSIBILITY MAP | FINAL FROZEN RULES |
| --- | --- | --- |
| frc.robot ¦- Main / Robot / RobotContainer / Constants ¦- commands/ controls/ subsystems/ ¦- io/ ¦ `- <mechanism>/ ¦ ¦- <Mechanism>IO ¦ ¦- <Mechanism>IOReal ¦ `- <Mechanism>IOSim ¦- telemetry/ ¦ ¦- RobotTelemetry ¦ `- <mechanism>/ ¦ ¦- <Mechanism>TelemetryFacade ¦ `- focused telemetry modules `- util/ deploy/pathplanner/ ¦- paths/ `- autos/ | controls = Driver intent commands = Robot actions / coordination subsystems = Mechanism API and state io = Hardware contract + vendor APIs telemetry = Observation and diagnostics util = Generic shared helpers only SOURCE OF TRUTH Hardware values originate in IO. Mechanism state originates in subsystems/estimators. Telemetry only publishes snapshots. Each IO owns a mechanism-specific Inputs snapshot. | RobotContainer: create, select, inject, configure. Commands receive the smallest dependency needed. Subsystems depend on IO interfaces only. IO never publishes NetworkTables. Telemetry never reads vendor devices or controls behavior. Facade hides internal telemetry modules. Constants.java is default; constants/ requires formal review. commands/auto coordinates; PathPlanner files live in deploy/. New mechanisms follow responsibility patterns, not file-by-file copying. Normal lessons extend the backbone; they do not redesign it. |

APPROVED FLOWS

| CONTROL Controller -> controls -> command -> subsystem -> IO -> hardware | OBSERVATION IO Inputs / mechanism state -> telemetry facade -> typed NT4 | FORBIDDEN Telemetry -> control ¦ IO -> NT ¦ RobotContainer -> periodic logic |
| --- | --- | --- |

STATUS: FINAL FROZEN AND LOCKED - NO FURTHER BACKBONE CHANGES