# Swerve Robot Hardware Map v2.0

## Robot Electronics

| Device | Model | CAN ID |
| --- | --- | --- |
| PDP | CTRE PDP | 0 |
| Pigeon2 | CTRE Pigeon2 | 20 |
| roboRIO | roboRIO 2.0 | - |

## Swerve Modules

| Module | Drive | Steer | Encoder |
| --- | --- | --- | --- |
| FL | 21 | 22 | 23 |
| FR | 24 | 25 | 26 |
| BL | 27 | 28 | 29 |
| BR | 30 | 31 | 32 |

## Geometry

| Parameter | Value |
| --- | --- |
| Wheel Diameter | 4.0 in |
| Wheel Radius | 2.0 in |
| Track Width | 21.5 in |
| Wheelbase | 21.5 in |

## Module Configuration

| Item | Value |
| --- | --- |
| Manufacturer | West Coast Products |
| Module Type | WCP Legacy |
| Drive Ratio | 6.75:1 (final measured/commissioned) |

## CAN Network

PDP 0
Pigeon2 20
FL 21/22/23
FR 24/25/26
BL 27/28/29
BR 30/31/32

Commissioning history: 7.85:1 was the initial provisional value - superseded by physical measurement. Repeated physical tests comparing 20 motor rotations with wheel rotations established the installed ratio as 6.75:1. The final status is 6.75:1 authoritative/current; 7.85:1 is obsolete historical data only.